Company Expectations.

- Operations-Research Methods

  - Inputs:

    - Resources (Machines, Cycle Times, Process Steps, Product Process Routing, Set-up-times, Quality Rates, Operator, Product Variants, etc.)

      - GSTC.xlsx

    - Constraints (Available Time per Machine, Available Time per Operator, Tool Change Times, etc.)

    - Production quantities (EDIs, weekly demands)

  - Outputs:

    - Standalone Tool with input methods, that is useable in the future by our colleagues

    - Production plan for period X, optimized for (maybe be able to select focus, what to optimize for)

      - Maximum Efficiency

      - Max Line Balancing

      - Min Tool Changes & Machine Setups

      - Optimal Batch and Buffer Sizes to reach these goals

      - Optimize Buffer for Raw Material per each Station -> Time Schedule for Material feeding

    - Production plan = Time schedule per machine containing, which process step of which product batch at what time

- Goal Result: Line Balancing & Reduce Setup Amounts & Batch amounts & VSM of DCC production

- Goal General: Create an application which can be used for other type of projects

System Architecture (Blue-Eye View)

1. Data Layer (Database)

- SQLite (for thesis & prototyping)

  - Easy, lightweight, file-based → no server required

- PostgreSQL (for company rollout, scalable option)

  - Multi-user, robust, integrates well with Power BI

Inputs stored here:

- Machine data (cycle times, availability, setup times)

- Operator availability

- Product routings

- Weekly demand (EDI forecasts)

- Quality & buffer limits

2. Backend Layer (Business Logic / OR Engine)

- Python (core computation engine)

  - Optimization: Pyomo, PuLP, OR-Tools

  - Scheduling: MILP / Constraint Programming models

  - Simulation: SimPy (if you want to test stochastic effects, like failures or variable setup times)

  - Data handling: Pandas, SQLAlchemy (to connect Data Base)

Functions handled here:

- Line balancing & bottleneck detection

- Production scheduling (time schedule per machine/operator)

- Setup/tool-change minimization

- Batch & buffer size optimization

- Material feeding time schedule

3. Frontend Layer (User Interface & Reporting)

Phase 1 – Prototyping (Thesis)

- Streamlit → lightweight web app

  - Upload Excel (GSTC.xlsx) or read from SQLite

  - Show Gantt charts, KPIs, dashboards

  - Interactive parameters (choose objective: efficiency, min setups, etc.)

Phase 2 – Scalable Future Use

- Power BI (Enterprise reporting)

  - Connects directly to PostgreSQL

  - Dashboards for production managers

  - Drill-down into efficiency, utilization, line balancing

4. User Layer

- Production planners, supervisors, and engineers interact with:

  - Streamlit tool (for optimization runs & scenarios)

  - Power BI dashboards (for monitoring & reporting)